# Wireless Mouse – Power Subsystem Requirements Specification

Battery: LiPo 3.7V 80mAh (UFX 302022)

Output Rail: 3.3V

Charging Interface: USB-C

## 1. System Overview

The power subsystem shall:
1. Accept 5V from USB-C.
2. Charge a single-cell LiPo battery (4.2V max).
3. Allow the mouse to operate while charging.
4. Provide a regulated 3.3V rail to all electronics.
5. Provide low-noise supply suitable for 2.4GHz RF.
6. Provide battery voltage monitoring.
7. Provide charge status indication LED.

## 2. Electrical Architecture

USB-C (5V) → ESD Protection → LiPo Charger IC → Battery (3.0–4.2V) → Low-Noise LDO (3.3V) → 3V3_MAIN → Ferrite Bead → 3V3_RF

## 3. USB-C Input Requirements

• Input Voltage: 5V ±5%
• Max Current Draw: ≤ 500mA
• USB Role: Sink only
• 5.1kΩ Rd resistors on CC1 and CC2
• TVS diode on VBUS
• TVS diode array for CC lines
• 1A resettable fuse on VBUS
• 10µF + 100nF input decoupling

## 4. LiPo Battery Requirements

• Nominal Voltage: 3.7V
• Full Voltage: 4.2V
• Cutoff Voltage: 3.0V
• Capacity: 80mAh
• Recommended Charge Current: 40–50mA
• Overcharge, overdischarge, and short-circuit protection required

## 5. Charger IC Requirements

• Single-cell Li-ion/LiPo support
• CC/CV charging algorithm (4.2V termination)
• Adjustable charge current (~50mA)
• Power path support
• Status output for LED
• Thermal regulation
• Standby current <50µA

## 6. Voltage Regulation Requirements

• LDO regulator
• Input range: 3.0–4.2V
• Output: 3.3V ±2%
• Max current: ≥100mA
• Dropout <200mV @30mA
• Quiescent current <10µA
• Output noise <100µVrms
• Ceramic capacitor stability

## 7. Output Power Domains

3V3_MAIN: MCU, optical sensor, buttons, LEDs
3V3_RF: RF section and crystal (isolated via ferrite bead)

## 8. Battery Monitoring

• ADC-based voltage divider measurement
• High impedance (>500kΩ total)
• 100nF filter capacitor
• Optional GPIO-controlled divider to reduce leakage

## 9. Charge Status LED

• Controlled by charger IC status pin
• LED current: 2–5mA
• Charging: LED ON
• Full: LED OFF or alternate indication

## 10. Current Consumption Budget

• Deep Sleep: 5–10µA
• Idle: 1–3mA
• Active: 5–10mA
• RF Burst: 15–25mA

## 11. Defined Power Nets

VBUS_5V
VBAT_RAW
BAT_SYS
3V3_MAIN
3V3_RF
GND